ATM1602B-FL-YBW--------------------- STANDARD CHARACTER LCD MODULES

* 5 x 8 with Cursor Dots                            * Transflective

* View Angle (6:00)                     		    * STN (Yellow green)

* 1/16 Duty, 1/5 Bias, 16CHAR x 2LINE               * Controller/Driver (S6A0069 )

* LED Backlight (Yellow green)     		    		* Operating Temp (-20℃~70℃)

Mechanical Specifications          Electrical Specifications

| ## Item | Description(mm) |  | ## Item | symbol | min | typ | max | unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size | 84(L)x44(W)x12.5(H) |  | Supply Voltage for Logic | VDD | 4.5 | 5.0 | 5.5 | V |
| V.A | 64.5(L)x16.4(W) |  | Supply Voltage for LCD | VLCD | 4.0 | 4.5 | 5.0 | V |
| Character Pitch | 3.51 x 5.75 |  | Supply Current for Logic | IDD |  | 1.0 | 2.0 | mA |
| Dot Size | 0.56*0.61 |  | Supply Current for LED | IF |  |  | 80 | mA |

Interface Signals---- J1

| Pin | Symbol | level | Description | Pin | Symbol | level | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Vss | 0V | Ground | 7~14 | DB0~DB7 | H/L | Data Bus Line |
| 2 | Vdd | +5V | Supply Voltage for Logic | 15 | A | -- | Back Light anode:+4.2V |
| 3 | V0 | -- | Operating Voltage for LCD | 16 | K | -- | Back Light cathode:0V |
| 4 | R/S | H/L | H: Data L: Instruction Code |  |  |  |  |
| 5 | R/W | H/L | H: Data Read L: Data Write |  |  |  |  |
| 6 | E | H,H-L | Enable Signal |  |  |  |  |