ATM4004A SERIES --------------------- STANDARD CHARACTER LCD MODULES

* 5 x 8 with Cursor Dots                            * Transflective 

* View Angle (6:00)                     

    * STN (Yellow green)                                                         

* 1/16 Duty, 1/5 Bias, 40CHAR x 4LINE               * Controller/Driver (S6A0069 )

* LED Backlight (Yellow green)     

    

* Operating Temp (-20℃~70℃)


[image: image1.wmf]1

2

17

18

K

A


Mechanical Specifications          Electrical Specifications
	Item
	Description(mm)
	
	Item
	symbol
	min
	typ
	max
	unit

	Size
	190(L)x54(W)x13.5(H)
	
	Supply Voltage for Logic
	VDD
	4.5
	5.0
	5.5
	V

	V.A
	3.53(L)x27.5(W)
	
	Supply Voltage for LCD 
	VLCD
	4.0
	4.5
	5.0
	V

	Character Pitch
	3.7*6.14
	
	Supply Current for Logic
	IDD
	
	1.0
	2.0
	mA

	Dot Size
	0.5*0.55
	
	Supply Current for LED
	IF
	
	
	80
	mA


Interface Signals---- J1 

	Pin 
	Symbol
	level
	Description
	Pin 
	Symbol
	level
	Description

	1
	Vss
	0V
	Ground
	7
	E1
	H,H-L
	Enable Signal

	2
	Vdd
	+5V
	Supply Voltage for Logic
	8
	E2
	H,H-L
	Enable Signal

	3
	V0
	--
	Operating Voltage for LCD
	9~16
	DB0~DB7
	H/L
	Data Bus Line

	4
	R/S


	H/L
	H: Data

L: Instruction Code
	17
	A
	--
	Back Light anode:+4.2V

	5
	R/W
	H/L
	H: Data Read L: Data Write
	18
	K
	--
	Back Light cathode:0V

	6
	VEE
	-5V
	Power Supply Voltage for LCD
	
	
	
	



_1152604228.dwg

